CCNA THEORY & PRACTICAL EXAM – SET 3

Duration: 2 Hours
Maximum Marks: 40
Theory: 25 Marks | Practical: 15 Marks

PART A – THEORY — 25 MARKS

Section A – Very Short Answer

Answer any 5 questions. (5 × 1 = 5 Marks)

- What is the purpose of the no command in Cisco IOS?

- Which memory contains the startup configuration?

- Which destination address does a router examine when forwarding an IP packet?

- What is meant by a remote network?

- What does /0 represent in IPv4 routing?

- What is the purpose of the show ip interface brief command?

Section B – Short Answer

Answer any 4 questions. (4 × 3 = 12 Marks)

- Explain the Access, Distribution and Core layers of a three-tier network architecture.

- Differentiate between enable password and enable secret.

- Explain how a router forwards a packet using its routing table.

- Explain the functions of CPU, ROM and NVRAM in a Cisco router.

- What is route summarization? Mention one benefit.

Section C – Descriptive Answer

Answer any 2 questions. (2 × 4 = 8 Marks)

- Explain default routing and the meaning of 0.0.0.0/0.

- Explain host routing and compare it with network routing.

- Describe the functions of CPU, RAM, ROM, Mini-IOS/ROMMON, Flash and NVRAM.

- Explain MPLS, SD-WAN and Enterprise VPN as WAN technologies.

PART B – PRACTICAL — 15 MARKS

Practical Question 1 – Static Routing — 6 Marks

Create a three-router topology:

R1 — R2 — R3

Use the following networks:

| Connection | Network | Addressing |
| --- | --- | --- |
| R1 LAN | 192.168.10.0/24 | R1 = 192.168.10.1 |
| R1–R2 | 10.0.0.0/30 | R1 = 10.0.0.1, R2 = 10.0.0.2 |
| R2 LAN | 192.168.20.0/24 | R2 = 192.168.20.1 |
| R2–R3 | 10.0.0.4/30 | R2 = 10.0.0.5, R3 = 10.0.0.6 |
| R3 LAN | 192.168.30.0/24 | R3 = 192.168.30.1 |

Tasks:

- Configure all router interfaces.

- Bring interfaces up.

- Configure appropriate static routes.

- Verify routing tables.

- Test end-to-end connectivity between the LANs.

Practical Question 2 – Default Routing — 5 Marks

On R3, configure a default static route toward R2.

Use:

- R3 → R2 next hop: 10.0.0.5

Tasks:

- Configure the default route.

- Verify it using show ip route.

- Demonstrate connectivity from the R3 LAN toward the remote network 192.168.10.0/24.

Practical Question – Floating Static Route — 5 Marks

Consider a network where R1 reaches the network 192.168.30.0/24 through R2 as the primary path and R3 as the backup path.

- Primary next hop: 10.0.0.2

- Backup next hop: 10.0.0.6

Tasks:

- Configure a normal static route to 192.168.30.0/24 through 10.0.0.2. (2 Marks)

- Configure a floating static route through 10.0.0.6 with an Administrative Distance of 10. (2 Marks)

- Verify the routing table and explain when the floating static route will be used. (1 Mark)