CCNA THEORY & PRACTICAL EXAM – SET 2

Duration: 2 Hours
Maximum Marks: 40
Theory: 25 Marks | Practical: 15 Marks

PART A – THEORY — 25 MARKS

Section A – Very Short Answer

Answer any 5 questions. (5 × 1 = 5 Marks)

- What is the purpose of the configure terminal command?

- What is the main purpose of Flash memory?

- What is the prefix length of a default route?

- What does the # symbol in a Cisco router prompt indicate?

- Name one command used to verify interface status.

- What function does DCE provide on a serial WAN connection?

Section B – Short Answer

Answer any 4 questions. (4 × 3 = 12 Marks)

- Explain the roles of ROM, Flash and NVRAM during router startup.

- Differentiate between User EXEC mode and Privileged EXEC mode.

- Explain host routes and default routes with their prefix lengths.

- Explain DTE and DCE in a leased-line WAN connection.

- What is Administrative Distance? Explain its role in route selection.

Section C – Descriptive Answer

Answer any 2 questions. (2 × 4 = 8 Marks)

- Explain the concept of Longest Prefix Match with a suitable example.

- Explain static routing and mention its advantages and disadvantages.

- Describe the purpose of Console, AUX and VTY lines.

- Explain PPP and the basic IOS commands used to configure PPP encapsulation.

PART B – PRACTICAL — 15 MARKS

Practical Question 1 – Static Routing + Load Balancing — 10 Marks

Create a topology with R1 and R2 connected using two separate point-to-point links.

Link 1

- R1: 10.0.0.1/30

- R2: 10.0.0.2/30

Link 2

- R1: 10.0.0.5/30

- R2: 10.0.0.6/30

Configure a LAN on R2:

- R2 LAN: 192.168.30.1/24

- Network: 192.168.30.0/24

Tasks:

- Configure both WAN links.

- Configure the R2 LAN interface.

- Configure two equal-cost static routes on R1 to 192.168.30.0/24:

  - Via 10.0.0.2

  - Via 10.0.0.6

- Verify that both routes are installed.

- Use show ip route to verify load balancing.

- Test connectivity from R1 to the R2 LAN.

Practical Question 2 – SSH Configuration — 5 Marks

Configure SSH access on a router with the following requirements:

- Hostname: R1-SSH

- Domain name: ccna.local

- Local username: admin

- Configure a password for the user.

- Generate RSA keys.

- Configure VTY lines to use the local user database.

- Permit SSH only.

- Verify the configuration using: show running-config